#### PROTOCOL: KIC Dye-Based Imaging_10.24.2025

#### Coating (96-well KIC plate)

- Matrigel coating  (1:1000, 100ul/well), aspirate leaving thin layer

  - 1:50 is the stock aliquot concentration, so take 1ml of 1:50 and dilute to 20ml final

- Add 200ul replating media WITH cells (70k) to middle of dish via 1ml pipette

- Next day, change the media out to one without ROCKi (2ml/dish)

- Image after 3 days minimum

#### Day-of-Imaging (ASSUMING 4+1 dishes)

- Prep 4 aliquots of Tyrode’s (for Calcium), pre-warmed

  - Rinse aliquot: 3 washes + 1 final set = ~50ml Tyrode’s

  - Dye aliquot = 100ul* #*1.1 wells…

- Rinse wells 1x with a set of pre-warmed Tyrode (100ul/well)

- To one set of 3, add appropriate dye mixture (2ml/dish) WITH HOECHST (1:5000)

  - Add FluoVolt 1000X (1x final = 10ul/10ml) + Powerload (100ul/10ml) or

    - Add in Powerload and FluoVolt initially, VORTEX & spin, then dilute with Tyrode’s, then add HOECHST (1:5000)

  - BeRST: (1.5uM final = 12ul/8ml) + Powerload (5ul/4ml) or

  - CalBryte: (1mM stock for 4uM final = 36ul 1mM CalBryte + Pluronic F-127 (0.04% final from 20% stock = 18ul) in 10ml Tyrode final

    - Reconstitute stock CalBryte 520 (46ul DMSO into 50ug) or 590 (41ul DMSO into 50ug)

    - *Vortex & spin down CalBryte with Pluronic before diluting in Tyrode’s to final

    - Add HOECHST (1:5000)

- Incubate 37C

  - FluoVolt = 25-30mins

  - BeRST = 30-60mins

  - Calbryte = 45mins

- OPTIONAL: While incubating, prep drug mixture for later challenge acutely at microscope (add directly on top, not aspirate)

  - ASSUMING 3.5ml FINAL/dish:

    - 4 dishes starting at 3ml so prep 3ml volume ISO (has extra)

    - Isoproterenol (4 dishes, add 500ul/dish, 100nM final from 100uM stock)

      - Add 3.5ul ISO/500ul buffer… for 4+1 dishes = 17.5ul ISO in 2.5ml buffer

- Rinse 2x

- Add final FluoroBrite DMEM or Tyrode (100ul) WITHOUT drug challenge

#### Using KIC

- Pre-warm KIC for 30mins with CO2 and Temperature control

- Set-up Scan

  - Select appropriate plate settings (P…Matrigel)

  - Select all wells of interest being used

  - Select middle of well

  - Adjust focus using DAPI

    - Focus Method = “Image Contrast & Surface Tracking” Select a well of interest  Set DAPI intensity to 30

    - “Find Surface” button“Advanced Settings” button Change upper range from 20 to 90umFull Focus MethodSave Values and Settings

    - Turn on light bulb  drag left-hand slider down to below 5000 to look for cells  turn off light bulb

  - Channel Acquisition Settings

    - Set for DAPI (Acquisition type = single image, intensity = 30, channel name = DAPI, Fluorochrome = DAPI)  “+” button to add next channel

    - Set for FITC or others (Acquisition type = kinetic series, intensity = variable, fluorochrome = FITC if FluoVolt)  Settings Button

      - Set Electrical Simulation

        - Voltage = 15V

        - Frames = 9800ms Delay, Frame Rate 100 Hz, Length 10s

        - Line 1 = 0 delay, 1 Hz frequency if APD or 0.5 Hz if calcium, 20 pulses, 9ms Pulse Width

        - KIC Display = Hi-Speed Write

      - Click Apply

- Place sample dish on scope stand without lid

- Tape down electrode to stand and tape wires  Connect wires to pacer (same colors)

- Start PACING (Enable), should hear chirping

  - Reducing pulse width to 9ms

    - Too long can re-excite cells and provoke double peaks

  - 15 Volts: Tune amplitude by threshold

    - 24V, which the lab uses, will KILL CELLS IN MINUTES

  - Use BIPHASIC, not a single pulse

  - Hz:

    - APD = 1.0

    - Ca2+ = 0.5

- TiPad,

  - E100 for eyes (turn on SolaPad and adjust %, usually 10 for GFP and 7 for TRITC)

    - Once focused (start at z-axis 5500), turn off SOLA

  - L100 for image acquisition

  - On SolaPad, ensure that auto-exposure if OFF

- Acquire tab  TimeLapse (Automatically)  Load Settings (if desired)  Name file  Apply & Run

  - IDEAL: 10 second duration @100FPS

    - Set to 1000 Loops, No Delay

- OPTIONAL: Draw ROIs while imaging or draw later

  - Draw ROIs (boxes or auto), use “alt” button to draw multiple

  - Measure all

  - Export pull down (Time Measurement Window) All to Excel  Export again

  - On Excel file, name the newly imported sheet, make new sheet

  - Continue imaging

#### RECORD:

#### 8/11/2025, 4 dishes + ISO, Calbryte 590, in 1hr 15mins

#### SHUT DOWN NIKON and PC

- 3 boxes on left first

- 2 extension blocks on wall

- Exit program and turn off PC

#### DRUG STRESSORS

#### 1. For APD (FluoVolt)

- Isoproterenol (ISO): β-adrenergic agonist

  - Purpose: Reveals APD shortening, arrhythmias, or repolarization defects

  - Dose: 100 nM

  - Time: Acute application (5–10 min pre-recording)

  - Expected Result:

    - Normal CMs: Shortened APD, enhanced contractility

    - LMNA mutants: Abnormal APD response (either exaggerated or blunted), possibly early afterdepolarizations

#### 2. For Calcium Handling (Fluo-4)

- Isoproterenol (ISO): β-adrenergic agonist

  - Purpose: Reveals APD shortening, arrhythmias, or repolarization defects

  - Dose: 100 nM

  - Time: Acute application (5–10 min pre-recording)

  - Expected Result:

    - Normal CMs:

    - LMNA mutants:

- Optional Add-on:

  - Thapsigargin (SERCA inhibitor, 1 µM): Used to block SR reuptake and isolate leak dynamics (e.g., to probe for CaMKII-RYR2 axis involvement)

  - Ryanodine (low dose ~100 nM): Stabilizes RyR2 channels; helps assess leak vs. overload

| Condition | Baseline | + Isoproterenol (100nM) |
| --- | --- | --- |
| H222P CRISPR KI | ✅ | ✅ |
| H222P SupRep | ✅ | ✅ |
| R541C SupRep | ✅ | ✅ |
| Untreated iPSC-CMs | ✅ | ✅ |