Calcium Transient Analysis for iPSC-Cardiomyocytes

Overview

This Python script provides a robust and automated workflow for analyzing calcium transient data obtained from live-cell imaging of induced pluripotent stem cell-derived cardiomyocytes (iPSC-CMs). It is designed to process time-series fluorescence data, typically from .nd2 files exported into an Excel format, and extract key physiological parameters from individual regions of interest (ROIs).

The script performs normalization, photobleaching correction, and applies a sophisticated peak detection algorithm to identify and quantify individual calcium transients. It then calculates a suite of standard metrics used in cardiac electrophysiology to characterize calcium handling.

This tool is particularly useful for researchers studying cardiomyocyte function, maturation, disease modeling (such as cardiomyopathies), and for assessing the effects of pharmacological compounds.

Getting Started

Prerequisites

- Python 3.x: Ensure you have a recent version of Python installed.

- Required Libraries: The script relies on several Python libraries. You can install them using pip:
pip install numpy pandas matplotlib scipy openpyxl xlsxwriter

Input File Preparation

- Export Data: Export your time-series fluorescence data (e.g., from NIS-Elements) into a Microsoft Excel (.xlsx) file.

- Format Excel Sheets:

- Each sheet in the Excel file should represent a single recording or sample.

- The sample name (e.g., the original .nd2 filename) should be in cell A1.

- The data should start on the second row, with headers in row 2.

- The first column should be the time data (in seconds).

- Subsequent columns should contain the fluorescence intensity data for each ROI. The ROI columns must contain "Mono" in their header name (e.g., "#1 (Mono - Tx Red)").

Usage

- Configure the Script: Open the Calcium Calbryte v1.ipynb script and modify the excel_path variable to point to your input Excel file.
# Please update this path to your actual file path
excel_path = r"C:\Path\To\Your\DataFile.xlsx"

- Adjust Parameters (Optional): You can fine-tune the analysis by adjusting the parameters in the "Config" section of the script. See the "Customization" section below for more details.

- Run the Script: Execute the cells in the Jupyter Notebook. The script will process each sheet in the specified Excel file.

Features

- Automated Batch Processing: Analyzes all sheets within a single Excel workbook, treating each as a separate experiment.

- Dynamic Peak Detection: Employs a robust algorithm to dynamically identify the start of depolarization and the peak of each calcium transient.

- Signal Processing:

- Normalization: Normalizes fluorescence signals to the minimum fluorescence (Fmin) to represent data as ΔF/Fmin.

- Smoothing: Applies a Gaussian filter to reduce noise in the signal.

- Photobleaching Correction: Implements an interpolation-based method to correct for signal drift due to photobleaching.

- Data Filtering: Includes optional filters to remove artifacts and select for physiologically relevant events based on upstroke time and transient duration.

- Comprehensive Output: Generates a detailed Excel report with raw data, summary statistics, and publication-quality plots.

Output

The script generates a new Excel file named [OriginalFileName] batch analysis.xlsx in the same directory as the input file. This output file contains three sheets:

- Summary: A detailed table listing all the calculated metrics for every valid calcium transient detected in each ROI.

- Mean_Values: A summary table showing the mean and standard deviation for each metric, grouped by sample and ROI.

- Plots: A sheet containing plots for each ROI, visualizing the processed signal, the detected peaks, and depolarization start times. This allows for easy visual verification of the analysis.

Metrics Explained

The script calculates the following standard metrics for calcium handling:

| Metric | Description |
| --- | --- |
| Amplitude | The magnitude of the calcium transient (ΔF/Fmin), from baseline to peak. |
| CTD50_s | Calcium Transient Duration at 50% decay: The time (in seconds) from depolarization to 50% repolarization. |
| CTD90_s | Calcium Transient Duration at 90% decay: The time (in seconds) from depolarization to 90% repolarization. |
| Peak_to_50%_Decay_Time_s | The time (in seconds) from the peak of the transient to 50% decay. |
| Peak_to_90%_Decay_Time_s | The time (in seconds) from the peak of the transient to 90% decay. |
| Upstroke_Time_s | The time (in seconds) from depolarization start to the peak of the transient. |
| Vmax | The maximum velocity of the calcium transient upstroke (d(ΔF/Fmin)/dt). |

Customization

You can adjust the following parameters in the script's "Config" section to tailor the analysis to your specific data:

- band_factor: Controls the stringency of the filter for CTD90. A smaller value makes the filter more strict.

- upstroke_min: Sets the minimum upstroke time for a transient to be considered valid.

- APPLY_FILTERS: A boolean (True/False) to toggle the data filtering on or off.

- GREY_OUT_UNFILTERED: A boolean (True/False) to control whether filtered-out events are shown in grey on the plots.

Contact

For questions, suggestions, or to report issues, please contact Trung Huynh at huynh.trung@mayo.edu